# LM2596 Module Measurement Points and Accessibility

## Objective

This document identifies the accessible measurement points on a commercial LM2596 buck converter module and outlines which electrical quantities can be directly measured using the available test equipment.

## Overview

The LM2596 is a monolithic switching buck regulator operating at approximately 150 kHz. Unlike discrete power converter implementations, the switching transistor is integrated within the IC package and is not externally accessible. As a result, several internal device quantities commonly measured in power electronics laboratories cannot be directly observed on the module.

The purpose of this document is to identify accessible nodes for converter characterization and establish expectations for future measurements.

# Accessible Measurement Nodes

## Input Voltage (Vin)

### Probe Location

- VIN+ screw terminal

- Positive terminal of the input capacitor

### Reference

- VIN− terminal

- Ground plane

### Expected Behavior

A nearly constant DC voltage supplied by the FNIRSI DPS-150.

### Planned Measurements

- Input voltage verification

- Input regulation

- Input transient behavior

## Output Voltage (Vout)

### Probe Location

- VOUT+ screw terminal

- Positive terminal of the output capacitor

### Reference

- VOUT− terminal

- Ground plane

### Expected Behavior

A regulated DC voltage with superimposed switching ripple.

### Planned Measurements

- Output regulation

- Load regulation

- Line regulation

- Efficiency calculations

## Output Voltage Ripple (ΔVout)

### Probe Location

- Directly across the output capacitor terminals

### Reference

- Output capacitor negative terminal

### Expected Behavior

A small periodic ripple synchronized with the switching frequency.

### Planned Measurements

- Peak-to-peak ripple voltage

- RMS ripple voltage

- Ripple variation with load current

### Notes

Measurements should be performed using:

- AC coupling

- Short probe ground connection

- Minimum probe loop area

## Switch Node Voltage (VSW)

### Probe Location

Any node electrically common with:

- LM2596 switch pin

- Schottky diode cathode

- Inductor input terminal

### Reference

- Ground

### Expected Behavior

A pulsed waveform transitioning between approximately 0 V and Vin at the converter switching frequency.

### Planned Measurements

- Switching frequency

- Duty cycle

- Switching waveform shape

- Ringing and overshoot

## Feedback Voltage (VFB)

### Probe Location

- LM2596 feedback pin

- Resistor divider midpoint

### Reference

- Ground

### Expected Behavior

The LM2596 regulates the feedback pin to approximately:

VFB≈1.23 V

during normal operation.

### Planned Measurements

- Verification of regulation

- Feedback network behavior

- Fault diagnosis

# Grounding Strategy and Measurement References

## Ground Topology

Based on the LM2596 converter schematic, the module utilizes a common ground system. The following nodes are electrically connected:

- IN− terminal

- OUT− terminal

- Input capacitor negative terminal

- Output capacitor negative terminal

- Schottky diode return path

- LM2596 ground pin

As a result, all oscilloscope measurements may be referenced to the module common ground.

## Ground Continuity Verification

Prior to powered measurements, continuity should be verified between the following nodes using a digital multimeter:

| Node 1 | Node 2 |
| --- | --- |
| IN− | OUT− |
| IN− | Input capacitor negative terminal |
| OUT− | Output capacitor negative terminal |
| IN− | LM2596 ground node |

Successful continuity measurements confirm the presence of a common ground reference throughout the converter.

## Oscilloscope Ground Reference

For all measurements, the oscilloscope ground lead should be connected to the converter common ground.

Recommended attachment points include:

- OUT− terminal

- IN− terminal

- Negative terminal of the output capacitor

Whenever possible, the ground connection should remain fixed while only the probe tip is moved between measurement nodes.

This approach reduces setup complexity and minimizes the likelihood of probing errors.

# Recommended Probe Locations

| Measurement | Probe Tip Location | Ground Reference |
| --- | --- | --- |
| Input Voltage (Vin​) | IN+ terminal | IN− terminal |
| Output Voltage (Vout​) | OUT+ terminal | OUT− terminal |
| Output Ripple (ΔVout) | Output capacitor positive terminal | Output capacitor negative terminal |
| Switch Node Voltage (VSW​) | Inductor input terminal or diode cathode | Common ground |
| Feedback Voltage (VFB​) | Feedback divider midpoint | Common ground |

# Output Ripple Measurement Considerations

Output ripple measurements are particularly sensitive to probe grounding technique.

To minimize measurement artifacts:

- Use the shortest possible ground connection.

- Avoid long alligator-style ground leads when measuring ripple.

- Minimize probe loop area.

- Probe directly across the output capacitor whenever possible.

- Use AC coupling to isolate the ripple component from the DC output voltage.

Poor grounding practices may introduce additional noise and ringing that do not originate from the converter itself.

## Expected Measurement Limitation

Because the converter operates as a switching regulator, some measured noise may originate from:

- Probe loop inductance

- Ground lead inductance

- Environmental electromagnetic interference

- Oscilloscope noise floor

Therefore, measured ripple should always be interpreted in the context of the measurement setup.

### Engineering Note

For this project, maintaining a consistent ground reference is more important than maximizing the number of accessible probe locations. Since the LM2596 module is a highly integrated converter, characterization efforts will focus primarily on converter-level behavior (input/output performance, ripple, regulation, and switching characteristics) rather than internal transistor-level measurements.

# Non-Accessible Electrical Quantities

## Gate-to-Source Voltage (VGS)

### Accessibility

Not directly measurable.

### Reason

The switching MOSFET is integrated within the LM2596 package and its gate node is not externally available.

### Alternative Observation

Switching behavior may be inferred from the switch-node waveform.

## Drain-to-Source Voltage (VDS)

### Accessibility

Not directly measurable.

### Reason

The internal MOSFET terminals are not exposed to the user.

### Alternative Observation

Switch-node measurements provide indirect visibility into switching operation.

## Inductor Current Ripple (ΔIL)

### Accessibility

Not directly measurable with the current hardware configuration.

### Reason

The module does not provide:

- A current-sense resistor

- A current-monitor output

- Dedicated current measurement test points

### Alternative Method

Inductor ripple current may be estimated using converter operating conditions:

ΔIL=Vin-VoutDLfs

where

D≈VoutVin

and:

- L = Inductance

- fs = Switching frequency

Estimated ripple current may later be compared qualitatively against measured output ripple voltage.

# Summary

The LM2596 module allows direct measurement of:

- Input voltage (Vin)

- Output voltage (Vout)

- Output ripple (ΔVout)

- Switch-node waveform (VSW)

- Switching frequency (fs)

- Duty cycle (D)

- Feedback voltage (VFB)

The following quantities are not directly accessible:

- Gate-to-source voltage (VGS)

- Drain-to-source voltage (VDS)

- Inductor current ripple (ΔIL)

These limitations arise from the integrated nature of the LM2596 switching transistor and the absence of dedicated current-sense circuitry on the module.

## Conclusion

The LM2596 module provides sufficient access to characterize overall converter performance, regulation behavior, switching operation, and output ripple. While internal transistor voltages and inductor current ripple cannot be directly observed, the available measurement points enable meaningful validation of converter operation and establish a foundation for more advanced power electronics characterization projects.